Build the 16-bit Register chip in HDL. Show read/write operations for three sample inputs and capture simulation output.

CHIP Register {

IN in[16], load;

OUT out[16];

PARTS:

Bit(in=in[0],  load=load, out=out[0]);

Bit(in=in[1],  load=load, out=out[1]);

Bit(in=in[2],  load=load, out=out[2]);

Bit(in=in[3],  load=load, out=out[3]);

Bit(in=in[4],  load=load, out=out[4]);

Bit(in=in[5],  load=load, out=out[5]);

Bit(in=in[6],  load=load, out=out[6]);

Bit(in=in[7],  load=load, out=out[7]);

Bit(in=in[8],  load=load, out=out[8]);

Bit(in=in[9],  load=load, out=out[9]);

Bit(in=in[10], load=load, out=out[10]);

Bit(in=in[11], load=load, out=out[11]);

Bit(in=in[12], load=load, out=out[12]);

Bit(in=in[13], load=load, out=out[13]);

Bit(in=in[14], load=load, out=out[14]);

Bit(in=in[15], load=load, out=out[15]);

}

BOOLEAN EQUATION

Qnext​=load⋅in+load⋅Q

TRUTH TABLE

| load | in[16] | Operation | out[16] |
| --- | --- | --- | --- |
| 0 | XXXXX | Hold/Read | Previous value |
| 1 | 0000...0000 | Write 0 | 0000...0000 |
| 1 | 1111...1111 | Write 1s | 1111...1111 |

Simulation Output

DIAGRAM

load

│

▼

┌────────────────┐

in[16] ───►│                │

│    Mux16       │────► next[16]

out[16] ────►│                │

└────────────────┘

│

▼

┌───────────┐

│   DFF16   │

│ 16 DFFs   │

└─────┬─────┘

│

▼

out[16]

​

Build the 8-register Register File chip in HDL. Trace read/write operations for at least four different addresses. Show all register states after each write.

CHIP RegisterFile {

IN in[16], load, address[3];

OUT out[16];

PARTS:

// Select one register for writing

DMux8Way(in=load, sel=address,

a=load0,

b=load1,

c=load2,

d=load3,

e=load4,

f=load5,

g=load6,

h=load7);

// Eight 16-bit registers

Register(in=in, load=load0, out=out0);

Register(in=in, load=load1, out=out1);

Register(in=in, load=load2, out=out2);

Register(in=in, load=load3, out=out3);

Register(in=in, load=load4, out=out4);

Register(in=in, load=load5, out=out5);

Register(in=in, load=load6, out=out6);

Register(in=in, load=load7, out=out7);

// Select one register for reading

Mux8Way16(

a=out0,

b=out1,

c=out2,

d=out3,

e=out4,

f=out5,

g=out6,

h=out7,

sel=address,

out=out);

}

Simulation Output

BLOCK DIAGRAM

address[3]

│

┌──────────┴──────────┐

│                     │

▼                     ▼

┌────────┐          ┌────────────┐

load ──────►│ DMux   │          │    Mux     │────► out[16]

│ 8-Way  │          │  8-Way 16  │

└───┬────┘          └──────▲─────┘

│                      │

┌─────────┼─────────┐            │

▼         ▼         ▼            │

┌────┐    ┌────┐    ┌────┐         │

in ─►│ R0 │    │ R1 │ ...│ R7 │─────────┘

│16bit│   │16bit│   │16bit│

└────┘    └────┘    └────┘

TRUTH TABLE

| load | address | Operation |
| --- | --- | --- |
| 0 | Any | No register is written |
| 1 | 000 | Write to R0 |
| 1 | 001 | Write to R1 |
| 1 | 010 | Write to R2 |
| 1 | 011 | Write to R3 |
| 1 | 100 | Write to R4 |
| 1 | 101 | Write to R5 |
| 1 | 110 | Write to R6 |
| 1 | 111 | Write to R7 |

Boolean Equations

The DMux generates the individual load signals:

L0=load⋅A2⋅A1⋅A0
L1=load⋅A2⋅A1⋅A0

Similarly, each address activates exactly one register.

The output can be represented as:

out=MUXR0R1R2R3R4R5R6R7

where the 3-bit address selects the required register.

Build the 1-bit DFF-based register (Bit chip) in HDL. Demonstrate read and write timing with a clock cycle trace showing tick-tock behavior.

CHIP Bit {

IN in, load;

OUT out;

PARTS:

Mux(a=out, b=in, sel=load, out=next);

DFF(in=next, out=out);

}

Simulation Output

Boolean Equation

BLOCK DIAGRAM

load

│

▼

┌───────────┐

in ──────►│           │

│    MUX    │────► next

out ───────►│           │

└───────────┘

│

▼

┌─────────┐

│   DFF   │

│         │

└────┬────┘

│

▼

Out

TRUTH TABLE

| load | in | Operation | out(next) |
| --- | --- | --- | --- |
| 0 | 0 | Hold | Previous out |
| 0 | 1 | Hold | Previous out |
| 1 | 0 | Write 0 | 0 |
| 1 | 1 | Write 1 | 1 |